### Rule 1 — Box-to-box Euclidean distance as the matching metric

Why:
A component can have several tags near it. We need a reliable way to determine which tag is physically closest to which component instead of simply checking whether a tag appears somewhere within a general search radius.

Intent:
Measure the actual distance between each component's bounding box and each tag's bounding box.

This gives us a numerical distance for every possible component–tag pair:

The smaller the distance, the closer the tag is to the component.

If the two boxes overlap or touch, the distance is 0.

For example:

Component              Tag

┌──────────┐

│          │

│          │  5 pt  ┌─────────┐

└──────────┘───────►│ TE-1203 │

└─────────┘

The distance between them is 5 pt.

Formula:
For a component box:

B=(x0,y0,x1,y1)B=(x_0,y_0,x_1,y_1)

and a tag box:

T=(tx0,ty0,tx1,ty1)T=(tx_0,ty_0,tx_1,ty_1)

First calculate the horizontal and vertical gaps:

dx=max⁡(x0−tx1,  tx0−x1,  0)dx=\max(x_0-tx_1,\;tx_0-x_1,\;0) dy=max⁡(y0−ty1,  ty0−y1,  0)dy=\max(y_0-ty_1,\;ty_0-y_1,\;0)

Then calculate the shortest distance between the two boxes:

d(B,T)=dx2+dy2d(B,T)=\sqrt{dx^2+dy^2}

So:

- Overlapping boxes → distance = 0

- Touching boxes → distance = 0

- Separated horizontally → distance = horizontal gap

- Separated vertically → distance = vertical gap

- Separated diagonally → distance = straight-line gap

Implementation:
Instead of calculating the distance one component and one tag at a time, calculate all distances at once and store them in a distance matrix.

For example, if there are 3 components and 4 tags:

Tags

T1    T2    T3    T4

Components

C1           0     18    42    75

C2           25    5     12    60

C3           80    30    0     15

Each number tells us how far that particular tag is from that particular component.

This matrix is then reused by the later rules:

Rule 1 → Calculate distances

↓

Rule 2 → Remove incompatible tag/component pairs

↓

Rule 3 → Find closest valid tags

↓

Rule 4 → Make sure each tag is used only once

So, in the simplest terms:

Rule 1 = "How physically close is every tag to every component?"

Yes. Here is a simpler, more understandable version of Rule 2, while keeping it technically precise:

### Rule 2 — Type-consistency filter via ISA prefix mapping

Why:
A tag should not be matched to a component just because it is nearby. The tag must also make sense for that component's type. This prevents cases where a physically closer but incorrect tag, such as PSV-1203, gets assigned to a Temperature or Flow component.

Intent:
Before comparing distances, check whether the tag is compatible with the component type. Reject tags that don't belong to the right ISA function category, even if they are physically closer.

For example:

Temperature Primary Element

↓

Accept: TE, TT, TI, etc. → Temperature

Reject: FE, FT            → Flow

Reject: PSV               → Pressure/Safety

Formula:
For each tag, determine its ISA category:

ϕ(t)=ISA category of tag t\phi(t) = \text{ISA category of tag }t

For each component, determine which categories are allowed:

Ψ(c)=allowed ISA categories for component c\Psi(c) = \text{allowed ISA categories for component }c

A tag is valid for a component only when:

consistent(c,t)={1if ϕ(t)∈Ψ(c)0otherwise\text{consistent}(c,t) = \begin{cases} 1 & \text{if } \phi(t) \in \Psi(c)\\ 0 & \text{otherwise} \end{cases}

In simple terms:

If the tag's category matches a category allowed for the component → keep it. Otherwise → reject it.

Implementation:

- Extract the ISA prefix from each tag, e.g. TE-1203 → TE, FE-1204 → FE, PSV-1203 → PSV.

- Map that prefix to an ISA category using the existing ISA_FUNCTION_GROUPS / ISA_MODIFIER mappings.

- Create a lookup table that says which ISA categories are allowed for each CE component type.

- Compare the tag category with the component's allowed categories.

- Set incompatible component-tag pairs to ∞ in the distance matrix so they cannot be selected by the later nearest-distance matching.

- If process_tags.csv already provides a reliable tag_type, use that classification first; only fall back to the raw ISA-prefix mapping when necessary.

So the overall logic becomes:

Rule 2 = "Is this tag the right kind of tag for this component?"
Rule 3 = "Among the right kinds of tags, which one is closest?"

### Rule 3 — Nearest valid candidate

Why:
After Rule 2 removes tags that are not compatible with the component type, there may still be several valid tags nearby. We need a consistent way to choose between them.

Intent:
From the tags that passed Rule 2, choose the tag that is physically closest to the component.

In simple terms:

Ignore the wrong types. Among the remaining correct types, pick the closest one.

For example:

Component:

Flow Primary Element

Nearby tags:

FE-1204  → Flow       → 10 pt away  ✓

FT-1205  → Flow       → 25 pt away  ✓

PSV-1203 → Pressure   →  5 pt away  ✗

Rule 2 rejects PSV-1203 because it is the wrong type.

Rule 3 then compares only:

FE-1204 → 10 pt

FT-1205 → 25 pt

So:

FE-1204 wins because it is the closest valid tag.

Formula:

For each component cc, find the tag tt with the smallest distance among the compatible tags:

match(c)=arg⁡min⁡tconsistent(c,t)=1d(c,t)\text{match}(c) = \arg\min_{\substack{t\\\text{consistent}(c,t)=1}} d(c,t)

In simple terms:

match = the closest tag that is allowed for this component.

Implementation:
Use the distance matrix created in Rule 1.

- Take one component's row from the distance matrix.

- Ignore the tags that Rule 2 marked as incompatible (∞).

- Find the smallest remaining distance using argmin.

- Assign that tag as the component's candidate match.

For example:

TE-1203   FE-1204   FT-1205   PSV-1203

Temperature       15        ∞         ∞          ∞

Flow              ∞         10        25         ∞

For the Flow Primary Element, the smallest valid distance is 10.

Therefore:

Flow Primary Element → FE-1204

One important point: Rule 3 only finds the nearest candidate independently for each component. It does not yet prevent two components from choosing the same tag. That's the problem Rule 4 — global exclusivity — solves.

### Rule 4 — Global exclusivity (one tag can belong to only one component)

Why:
Rule 3 finds the closest valid tag for each component independently. This can cause two or more components to choose the same tag.

For example:

Component A ── 5 pt ──┐

├── FE-1204

Component B ── 8 pt ──┘

If we simply run Rule 3:

Component A → FE-1204 ✓

Component B → FE-1204 ✓

But one tag cannot represent two different components.

Intent:
Make the matching one-to-one:

Each component can have at most one tag, and each tag can be assigned to at most one component.

When multiple components are competing for the same tag, the overall assignment should choose the combination of matches that gives the smallest total distance.

For example:

FE-1204    FT-1205

Component A         5          20

Component B         8          10

Simply choosing the nearest tag for each component would give:

Component A → FE-1204  (5)

Component B → FE-1204  (8)   ✗ Same tag

Instead, global assignment considers both components together:

Component A → FE-1204  (5)

Component B → FT-1205  (10)

Total distance:

5+10=155+10=15

This is better than leaving Component B with a duplicate assignment.

Formula:

Find a set of component–tag pairs MM that minimizes the total distance:

min⁡∑(c,t)∈Md(c,t)\min \sum_{(c,t)\in M} d(c,t)

while making sure:

each component appears at most once\text{each component appears at most once}

and

each tag appears at most once\text{each tag appears at most once}

In simple terms:

Find the best overall set of matches without using the same component or tag twice.

Implementation:

Use the distance matrix from Rule 1, after Rule 2 has removed incompatible pairs.

Then use:

scipy.optimize.linear_sum_assignment

(the Hungarian algorithm) to perform the global assignment.

The process becomes:

Rule 1

Calculate distances

↓

Rule 2

Remove incompatible pairs

↓

Rule 3

Identify closest valid candidates

↓

Rule 4

Look at ALL components together

↓

Prevent duplicate tag assignments

↓

Choose the combination with

the lowest total distance

So the key difference is:

Rule 3 asks: "What's the closest valid tag for this component?"

Rule 4 asks: "What's the best overall set of matches when all components and tags have to share the same pool?"

### Rule 5 — Exclude Line Tag from the candidate pool

Why:
Not every tag in the P&ID is meant to identify a component. A Line Tag identifies a pipe/process line, not an instrument or equipment component. If we allow Line Tags into this matching process, a component might incorrectly get assigned a nearby line number.

For example:

Component

│

│  5 pt

▼

┌─────────┐

│ 120-101 │  ← Line Tag

└─────────┘

The line tag may be the closest piece of text, but it is not the component's tag.

Intent:
Keep the component ↔ tag matching separate from the line ↔ tag matching.

For this process:

Only consider tags that can actually identify a component.

Line Tag is handled separately in the later line-tag association step.

Formula:

If TT is the complete set of extracted tags, create a smaller candidate pool:

Tcandidate={t∈T∣t.tag_type≠"Line Tag"}T_{\text{candidate}} = \{t\in T \mid t.\text{tag\_type}\neq\text{"Line Tag"}\}

In simple terms:

Take all tags, remove the ones classified as Line Tag, and use whatever remains for component matching.

For example:

All extracted tags

↓

┌──────────────────────────────┐

│ TE-1203       Instrument     │ ✓

│ FE-1204       Instrument     │ ✓

│ PSV-1205      Instrument     │ ✓

│ 120-101       Line Tag       │ ✗

│ 120-102       Line Tag       │ ✗

└──────────────────────────────┘

↓

Candidate pool

TE-1203, FE-1204, PSV-1205

Implementation:
Before creating the distance matrix in Rule 1, filter the process_tags dataframe:

tag_type != "Line Tag"

Then build the component-to-tag distance matrix only using this filtered set.

So the overall idea is:

Rule 5 = "Don't let pipe/line numbers compete with actual component tags."

### Rule 6 — Two-pass radius: tight first, then widen

Why:
A large search radius can create too many possible matches, especially when several components and tags are close together. A tag that is 50–60 pt away might be technically "nearby" but could actually belong to something else.

We therefore want to prefer nearby, high-confidence matches and only look farther away when necessary.

Intent:
Use a small search radius first. If a component cannot find a valid tag within that small radius, give it a second chance using the larger radius.

In simple terms:

Look close first. If nothing suitable is found, look farther.

For example:

Component

│

├── 12 pt ── TE-1203   ✓

│

└── 50 pt ── TE-1208   ✓

If the tight radius is 25 pt:

Pass 1 (25 pt)

TE-1203 → 12 pt → considered ✓

TE-1208 → 50 pt → ignored

So TE-1203 gets matched immediately.

Now consider a component with no suitable tag nearby:

Component

│

└── 45 pt ── FE-1204

Pass 1:

25 pt radius

↓

No valid tag found

↓

UNMATCHED for Pass 1

Then Pass 2 expands the search:

60 pt radius

↓

FE-1204 → 45 pt → valid

↓

MATCH

### Formula

Define:

Rtight≈20–25 ptR_{\text{tight}}\approx20\text{–}25\text{ pt}

and:

Rwide=60 ptR_{\text{wide}}=60\text{ pt}

Pass 1:

d(c,t)≤Rtightd(c,t)\le R_{\text{tight}}

Run Rules 1–4 using only tags within the tight radius.

Pass 2:

For components that are still unmatched:

d(c,t)≤Rwided(c,t)\le R_{\text{wide}}

Run Rules 1–4 again, using only the remaining components and remaining tags.

In simple terms:

All components + tags

↓

PASS 1

≤ 25 pt

↓

High-confidence matches

↓

Remaining components/tags

↓

PASS 2

≤ 60 pt

↓

Additional matches

Implementation:

- Set the tight radius to around 20–25 pt.

- Run the complete matching process:

  - Rule 1 → calculate distances

  - Rule 2 → remove incompatible types

  - Rule 3 → find nearest valid candidates

  - Rule 4 → enforce one-to-one assignment

- Keep those matches.

- Identify components that are still unmatched.

- Remove tags that were already used.

- Run the same Rules 1–4 again, but with the 60 pt radius.

- Add the second-pass matches to the first-pass matches.

The important part is that Pass 2 does not reconsider successful Pass 1 matches.

So:

Rule 6 = "Take the easy, close matches first. Only search farther for components that still don't have a match."

### Rule 7 — Agreement flag against CE's existing tag field

Why:
The CE system has already assigned a tag to each component. Our new matching process gives us another independently calculated match. Comparing the two tells us where CE and our method agree and where they disagree.

This is useful because a disagreement doesn't automatically mean CE is wrong—it simply tells us:

"This component deserves a closer look."

Intent:
Compare the tag found by our new matching rules with the tag that CE already assigned.

For example:

Component: Flow Primary Element

CE's tag:       SP 112

Our match:      FE-1204

↓

DISAGREE

This tells us that the component is worth reviewing.

If they are the same:

Component: Temperature Primary Element

CE's tag:       TE-1203

Our match:      TE-1203

↓

AGREE

That gives us additional confidence in the existing CE result.

Formula:

agrees(c)=1[match(c)=c.tagCE]\text{agrees}(c) = \mathbb{1} \left[ \text{match}(c)=c.\text{tag}_{CE} \right]

In simple terms:

Our matched tag == CE's tag

↓

AGREE ✓

Our matched tag != CE's tag

↓

DISAGREE ⚠

Implementation:
After Rules 1–6 have finished:

- Take the tag assigned by our matching process.

- Take the existing tag value from the CE data.

- Compare the two.

- Add a new column such as tag_agrees.

For example:

| Component | CE tag | Our match | tag_agrees |
| --- | --- | --- | --- |
| Temperature Element | TE-1203 | TE-1203 | True |
| Flow Element | SP 112 | FE-1204 | False |
| Pressure Element | PT-1205 | PT-1205 | True |

Important:
We do not overwrite CE's existing tag field.

We simply add the comparison result, so we can later distinguish:

- Agreement → CE and our method found the same tag.

- Disagreement → our method found something different; review it.

- Unmatched → our method couldn't find a suitable tag.

So:

Rule 7 = "Did our matching method find the same tag that CE originally found?"

### Rule 8 — Track both unmatched sets explicitly

Why:
After the matching process is complete, some components may still have no assigned tag, and some tags may still have no assigned component. These two situations are different and should be tracked separately instead of being combined or discarded.

An unmatched component means:

"We found a component, but couldn't find a suitable tag for it."

An orphan tag means:

"We found a tag, but couldn't find a suitable component for it."

These lists are useful for identifying things that may have been missed by the detection or matching process.

Intent:
Keep two separate lists after the matching process:

- Unmatched Components — components that still have no tag.

- Orphan Tags — eligible tags that still have no component.

Importantly, we do not immediately consider a component unmatched after Pass 1. Rule 6 gives it a second chance using the wider radius.

Pass 1

25 pt radius

↓

Matched ───────────→ Keep

│

↓

Unmatched

↓

Pass 2

60 pt radius

↓

Matched ───────────→ Add to matches

│

↓

Still unmatched

Only the components that remain unmatched after both passes are recorded as final unmatched components.

Formula:

For components:

Unmatched Components=C−{c:c is matched after Pass 1 and Pass 2}\text{Unmatched Components} = C-\{c:c\text{ is matched after Pass 1 and Pass 2}\}

For tags:

Orphan Tags=Tpool−{t:t is matched after Pass 1 and Pass 2}\text{Orphan Tags} = T_{\text{pool}}-\{t:t\text{ is matched after Pass 1 and Pass 2}\}

In simple terms:

Run the matching process, including the wider second pass from Rule 6, and then whatever is still unused goes into the appropriate unmatched list.

Implementation:

- Run Pass 1 using the tight radius from Rule 6.

- Keep all successful matches.

- Identify components that were not matched.

- Run Pass 2 on those remaining components using the wider radius.

- Do not reuse tags that were already assigned during Pass 1.

- Add the successful Pass 2 matches to the final match list.

- After both passes are finished, identify the components that still have no match.

- Identify the candidate-pool tags that were never assigned.

- Write these as two separate CSV files:

  - unmatched_components.csv

  - orphan_tags.csv

For example:

85 components

↓

Pass 1 (25 pt)

↓

54 matched

31 unmatched

↓

Pass 2 (60 pt)

↓

+1 matched

30 still unmatched

And for tags:

110 candidate tags

↓

55 assigned to components

↓

55 orphan tags

The important point is:

"Unmatched" in Rule 8 means still unmatched after we've already tried both the tight and wide searches in Rule 6.

These final unmatched sets should then be reviewed separately, because they can reveal different problems—for example, an unmatched component may indicate a missed tag, while an orphan tag may indicate a component that CE failed to detect.

Rule 8 = "After we've tried both matching passes, keep track of everything that is still unmatched—components and tags separately—so we can investigate what was missed."